INTERNET OF THINGS (IOT) WEARABLE SAFETY DEVICE FOR MONITORING GAS EXPOSURE, HEART RATE, AND TEMPERATURE OF WORKERS

BY

IHESIULO AKUNNA COMFORT

(22CE032235)

A PROJECT SUBMITED TO THE DEPARTMENT OF PHYSICS, COLLEGE OF SCIENCE AND TECHNOLOGY, COVENANT UNIVERSITY OTA, OGUN STATE.

IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF THE BACHELOR OF SCIENCE (HONOURS) DEGREE IN INDUSTRIAL PHYSICS (ELECTRONIC AND IT APPLICATION)

NOVEMBER, 2025

CERTIFICATION

I hereby certify that this project was carried out by Ihesiulo Akunna Comfort in fulfillment of part of the requirements for the award of Bachelor of Science (B.Sc) degree, Industrial Physic (Electronic and IT Application) of the Department of Physics, College of Science and Technology, Covenant University, Ogun State, Nigeria.

| Dr. S. A. Akiwunmi |  |
| --- | --- |
| Supervisor | Signature and Date |
| Dr. Theophilus A. Adagunodo |  |
| Head of Department | Signature and Date |

DEDICATION

I thank God Almighty for his guidance, protection, grace, strength, courage, and provision throughout this project.

I want to also express gratitude to my family, Mr and Mrs kalu, my parents, who have supported me in every conceivable way, to my department, the Department of Physics, and to all my lecturers who have dedicated their lives to teaching and counselling and have played a significant role in this journey.

ACKNOWLEDGEMENTS

First and foremost, God is the source of my immense gratitude, as He enabled this work to be completed. I also want to thank my family for their unwavering provision and support. Similarly, I am thankful to Dr. S. A.  Akiwunmi, my supervisor, for his direction and oversight in this project and the head of the Department of Physics , for his guidance.

TABLE OF CONTENTS

CONTENTS		    PAGES

COVER PAGE		              i

CERTIFICATION	ii

DEDICATION	iii

ACKNOWLEDGEMENTS	iv

TABLE OF CONTENTS	v

ABSTRACT	xiv

CHAPTER ONE: INTRODUCTION	1

1.1	Background of the Study	1

1.2	Statement of the Problem	2

1.3	Aim and Objectives of he study	3

1.4	Justification of the Study	3

1.5	Scope of the Study	3

1.6	Limitation of the Study	4

CHAPTER TWO: LITERATURE REVIEW	8

2.1	Introduction	8

2.2	Concept of Internet of Things	8

2.3	Wearable Technology in Saftey and Health monitoring	9

2.4	Gas Detection and Environmental Monitoring Technology	14

2.5	Physiological Monitoring: heart rate and temperature sensing	15

2.6	Existing Wearable Safety Solution	16

2.7	Research	18

2.8	Summary	23

CHAPTER THREE: METHODOLOGY	24

3.1	Preamble	24

3.2	Research Design	24

3.3	System Overview	30

3.4	System Architecture	31

3.5	Hardware Component used	32

3.6	Software Development	33

3.7	Data flow Diagram 								  17

3.8 	Prototype Assemble and Testing						  20

3.9 	System Evaluation Criteria							  21

3.10	Summary 									  22

REFERENCES	72

CHAPTER ONE

# INTRODUCTION

  - Background of the Study

Industries such as mining, manufacturing, construction, and the oil and gas sector remain critical to global economic development. However, these environments are often classified as high-risk workplaces due to constant exposure to hazardous gases, unstable temperature conditions, and physically demanding tasks. Workers in these sectors may unknowingly inhale harmful gases such as carbon monoxide, methane, hydrogen sulfide, or ammonia, which can impair breathing, affect the nervous system, and in severe cases, lead to coma or death. Similarly, extreme environmental temperatures can lead to heat stroke, dehydration, exhaustion, or hypothermia depending on working conditions.

In the past decade, wearable technology has gained significant attention due to its potential to improve safety and preventive healthcare. The Internet of Things (IoT) has made it possible for devices to communicate autonomously, collect data, and trigger intelligent responses. When combined with wearable sensors, IoT enables practical and real-time monitoring of environmental hazards and physiological indicators such as heart rate and body temperature.

This project explores how IoT-based wearable devices can serve as reliable safety tools for industrial workers. The wearable safety device proposed in this study monitors gas concentration, heart rate, and temperature and sends the readings wirelessly to a central dashboard for supervisors or emergency teams. By offering live data and early warning notifications, the device can prevent life-threatening conditions and improve workplace safety compliance.

  - Statement of the Problem

Despite advancements in industrial policies and safety frameworks, many developing countries still rely on outdated, manual, or reactive safety monitoring systems. In several workplaces, gas detectors are installed only at fixed locations and not carried by personnel, which means individual workers may be exposed to toxic gases without being aware. In addition, health monitoring is rarely integrated into safety systems, so workers experiencing stress, irregular heart rate, overheating, or fatigue may collapse without warning.

Existing monitoring systems also lack remote data transmission, meaning supervisors are often unaware of hazardous situations until it is too late. Furthermore, many available commercial solutions are too expensive, complex, or lack IoT capability, making them unsuitable for local industries, especially in Africa and other developing regions.

This creates a clear gap and a need for an affordable, portable, intelligent wearable safety device capable of continuous environmental and health monitoring.

1.3 Aim and Objectives of the Study

The main aim of this project is to design, construct, and test an IoT-based wearable safety device capable of monitoring workers’ gas exposure levels, heart rate, and temperature in real-time.

To achieve this, the project will pursue the following specific objectives:

To design and build a wearable electronic device using appropriate sensors to measure gas concentration, body temperature, and heart rate.

To implement wireless data transmission using the ESP32 microcontroller and an IoT platform for remote access.

To develop a dashboard interface that visualizes data and provides alerts when abnormal readings are detected.

To test and evaluate the device under different working conditions to determine its efficiency, accuracy, and reliability.

  - Justification of the Study

This study is justified based on its potential contribution to industrial safety and preventive healthcare. Workplace accidents caused by toxic gas inhalation, high temperature exposure, or cardiovascular stress continue to be a serious concern globally. According to the International Labour Organization (ILO), over 2.3 million workers die each year due to workplace-related accidents and health complications—many of which are preventable.

Wearable IoT systems offer a modern solution by empowering both workers and supervisors with real-time health and environmental data. The device developed in this project is low-cost and uses readily available components, making it suitable for companies with limited safety budgets. The project also supports several global development goals including:

SDG 3: Good Health and Well-being

SDG 8: Decent Work and Economic Growth

SDG 9: Industry, Innovation and Infrastructure

By developing a working prototype, this study also contributes to ongoing research in smart safety systems, human factors engineering, and industrial IoT applications.

1.5 Scope of the Study

The project focuses on designing a functional prototype rather than a fully certified industrial product. The system includes three core sensing modules: an MQ-series gas sensor, a MAX30102 heart rate sensor, and an infrared temperature sensor (MLX90614). Data collected is processed by the ESP32 microcontroller and transmitted wirelessly to an IoT dashboard.

The scope does not include:

Industrial-grade calibration

Battery optimization for long-term field use

Rugged waterproof enclosures

Multi-gas classification algorithms

Future work may expand into these areas.

  - Limitations of the Study

Like most early-stage hardware designs, the system has technical and operational limitations. Sensor performance may be affected by noise, humidity, temperature variations, and calibration errors. Internet connectivity is required for real-time remote monitoring, meaning performance may degrade in underground or isolated locations.

The heart rate sensor may also be affected by sweat, movement, or tattoos on the skin. Finally, the project is constrained by limited time, available resources, and cost, hence full industrial certification and long-term testing were not feasible.

CHAPTER TWO

# LITERATURE REVIEW

2.1 Introduction

This chapter reviews existing studies and technological advancements related to wearable safety devices, IoT systems, environmental sensing, and physiological monitoring. It also discusses existing solutions and research gaps that justify the development of an IoT-based wearable safety device. Literature review is essential because it provides a theoretical foundation and shows how the present research builds on or diverges from previous work (Creswell, 2018).

2.2 Concept of Internet of Things (IoT)

The Internet of Things (IoT) refers to an interconnected system of devices equipped with sensors, connectivity, and processing ability to collect and exchange data autonomously (Ashton, 2009). IoT is built on a multilayer architecture typically consisting of the perception layer (sensors and microcontrollers), the network layer (Wi-Fi, Bluetooth, LoRaWAN, etc.), and the application layer (cloud platforms, dashboards, and analysis tools) (Madakam, Ramaswamy & Tripathi, 2015).

IoT systems have transformed numerous industries including agriculture, healthcare, logistics, smart home automation, and industrial safety. Due to low-power microcontrollers like ESP32 and embedded communication modules, IoT-based wearable devices are now feasible and increasingly adopted in occupational safety and monitoring applications (Al-Fuqaha et al., 2015).

2.3 Wearable Technology in Safety and Health Monitoring

Wearable technology refers to electronic systems that can be worn on the body to continuously monitor physiological and environmental parameters (Pantelopoulos & Bourbakis, 2010). Modern wearables integrate sensors, microcontrollers, power units, and wireless connectivity and are widely used in both personal health and workplace safety.

Studies have shown that wearables are significantly more effective in early hazard detection than manual or periodic monitoring methods (Lopez et al., 2018). In occupational settings, wearables help detect fatigue, heat stress, irregular heart rhythm, and exposure to harmful gases or extreme conditions (Dimitrov, 2016). This ability to provide real-time awareness makes wearable IoT systems a practical tool for safety-critical industries such as mining, construction, and oil and gas.

2.4 Gas Detection and Environmental Monitoring Technologies

Gas sensors are widely used in industrial safety systems to detect toxic or combustible gases. MQ-series sensors are common in research due to their affordability and ability to detect gas concentration using metal-oxide semiconductor sensing principles (Han, Zhang & Li, 2019). Harmful gases such as carbon monoxide, methane, and hydrogen sulfide are common in mining, petroleum refining, and chemical processing environments and can cause long-term health effects or death when inhaled at high concentrations (Occupational Safety and Health Administration, OSHA, 2022).

Environmental monitoring also includes temperature and humidity sensing. Exposure to excessive heat can cause fatigue, reduced concentration, dehydration, and in severe cases heat stroke (World Health Organization, 2021). Therefore, environmental monitoring is critical in industrial contexts to prevent health risks and maintain safe working conditions.

Table 2.1: Common Industrial Hazardous Gases and Their Effects

| Gas Type | Typical Source | Health Effects | Detection Threshold (Approx.) |
| --- | --- | --- | --- |
| Carbon Monoxide (CO) | Vehicle exhaust, furnaces, generators | Headache, dizziness, unconsciousness, death at high levels | 25–50 ppm |
| Methane (CH₄) | Mining, oil rigs, sewage systems | Highly flammable and explosive when concentrated | Detectable at 5,000–10,000 ppm |
| Hydrogen Sulfide (H₂S) | Sewage treatment, petroleum refining | Eye irritation, respiratory distress, nervous system damage | 10–20 ppm |
| Ammonia (NH₃) | Fertilizer plants, refrigeration systems | Skin irritation, coughing, lung damage at high concentrations | 25–35 ppm |
| Sulfur Dioxide (SO₂) | Chemical plants, combustion of sulfur-rich fuels | Throat irritation, breathing difficulty, risk of asthma | 2–5 ppm |

(Source: OSHA, 2022; Han et al., 2019)

Explanation to Follow the Table:

Industrial work environments expose individuals to different toxic gases, each with unique concentration thresholds and health implications. As shown in Table 2.1, some gases are harmful due to their toxic biological effects, while others are dangerous because of their flammability or explosive nature. The ability of wearable IoT devices to continuously monitor exposure levels ensures that workers receive early warnings before the concentration reaches hazardous limits. Such preventive monitoring has been proven effective in reducing workplace fatalities in high-risk industries (Fernandez et al., 2021).

Gas sensors such as the MQ series work based on changes in electrical resistance when exposed to target gases. The concentration of gas is typically determined using a logarithmic calibration curve expressed mathematically as:

Gas Concentration (ppm)= a (R_s)^b/(R_o)

Where:

( R_s ) = Sensor resistance in the presence of gas

( R_o ) = Sensor resistance in clean air

( a ) and ( b ) = Calibration constants obtained from the manufacturer or experimental calibration

This equation allows the sensor output to be converted into meaningful real-world concentration values rather than raw electrical readings.

2.5 Physiological Monitoring: Heart Rate and Temperature Sensing

Heart rate monitoring provides valuable insight into a worker’s physical condition, stress level, and fatigue (Raju & Pitchumani, 2020). Photoplethysmography-based devices like the MAX30102 measure heart rate by detecting variations in blood volume using optical signals (Rajput et al., 2022).

Body temperature monitoring is equally important in occupational health surveillance, particularly for workers in high-heat environments. Non-contact infrared sensors such as the MLX90614 allow safe temperature measurement without interfering with workers’ comfort (Sims, Adams & Murphy, 2021). The integration of these physiological sensing systems into IoT wearables supports continuous monitoring and early emergency detection.

Figure 2.1: General Structure of a Wearable IoT Monitoring System

[ Sensors ] → [ Microcontroller (ESP32) ] → [ Wireless Network (Wi-Fi/Bluetooth) ]

→ [ IoT Platform / Cloud Server ] → [ Dashboard / Mobile App ] → [ Alerts / Notifications ]

Figure 2.1: Flow representation of how sensor readings are processed, transmitted, and visualized in an IoT-based wearable monitoring system.

Suggested Explanation Below the Figure:

Figure 2.1 illustrates the general architecture of a wearable IoT monitoring device. The system begins with the data acquisition stage, where the sensors collect physiological and environmental readings. These values are processed locally by the microcontroller, such as the ESP32, before being transmitted wirelessly to an IoT platform. Once uploaded, the data can be visualized on a dashboard where supervisors, medical teams, or automated software can evaluate safety conditions in real time. Alerts may be triggered when readings exceed predefined thresholds, ensuring a proactive safety response rather than a reactive one (Lopez et al., 2018; Madakam et al., 2015).

Heart rate sensors, such as the MAX30102, utilize photoplethysmography (PPG) to measure blood volume fluctuations. The heart rate (in beats per minute) is computed using:

HR (bpm) = (60)/(T)

Where:

( T ) = Time interval between two successive pulse peaks (seconds)

Additionally, temperature readings from infrared sensors follow Stefan-Boltzmann’s law, which relates emitted infrared power to temperature:

P = \sigma A T^4

Where:

( P ) = Radiant energy emitted

( \sigma ) = Stefan-Boltzmann constant (5.67×10⁻⁸ W/m²K⁴)

( A ) = Surface area

( T ) = Temperature in Kelvin

This ensures precise non-contact temperature measurement suitable for wearable devices.

2.6 Existing Wearable Safety Solutions

Commercial wearable gas detectors and smart health monitors exist; however, most are expensive, require subscription platforms, or do not integrate multiple sensing parameters (Fernandez et al., 2021). Some devices only detect toxic gas levels while others are purely biometric, meaning they cannot provide full situational awareness (Park & Jayaraman, 2020).

Academic prototypes provide promising alternatives but often lack durability testing, long-term battery optimization, or cloud-based monitoring (Hussain et al., 2022). This gap demonstrates the need for a wearable device that is multi-functional, affordable, and suitable for developing economies.

2.7 Research Gap

Based on reviewed literature, several gaps are evident. Existing monitoring systems either lack IoT remote monitoring capabilities, do not combine environmental and biometric sensing, or are financially inaccessible to smaller industries (Fernandez et al., 2021; Hussain et al., 2022). There is therefore a need for a low-cost, wearable IoT-based safety device that provides multi-parameter monitoring in real-time.

2.8 Summary

This chapter reviewed published research relevant to IoT systems, wearable device development, environmental sensing, and biometric monitoring. The findings reveal that while wearable IoT systems exist, affordability, multi-sensor integration, and accessibility remain major challenges. These gaps justify the development of the proposed IoT-based wearable system for industrial safety and worker health monitoring.

CHAPTER THREE

# METHODOLOGY

3.1 Introduction

This chapter explains the methods, tools, components, and design processes used in developing the IoT-based wearable safety device. It also describes the system architecture, hardware configuration, software programming, testing procedure, and evaluation methods. A well-defined methodology ensures that the project is reproducible and allows others to evaluate the strength and reliability of the approach used.

3.2 Research Design

A hardware-software experimental research design was adopted for this project. The hardware component includes sensors, the ESP32 microcontroller, and power management modules. The software component involves programming the device, transmitting data wirelessly, and visualizing the measured values on an IoT dashboard. The design followed an iterative engineering approach, meaning the system was developed in stages, tested, and improved at each step.

3.3 System Overview

The wearable safety device integrates three types of sensors:

A gas sensor for detecting hazardous gases

A heart rate sensor for physiological monitoring

A temperature sensor for body temperature measurement

The sensor readings are processed by the ESP32 microcontroller, which acts as the central control unit. Data is transmitted wirelessly over Wi-Fi to an IoT platform where it is stored, analyzed, and displayed in real time.

3.4 System Architecture

The system architecture consists of five major layers:

Sensing Layer – collects raw environmental and physiological data from the sensors.

Processing Layer – uses ESP32 to convert sensor signals to digital readings and perform initial filtering.

Communication Layer – transmits data to the cloud using built-in Wi-Fi communication.

Cloud/IoT Layer – stores and visualizes data using a dashboard or mobile application.

User Interface Layer – enables supervisors or workers to monitor readings and receive alerts.

Figure 3.1: System Architecture Diagram

[Sensors] → [ESP32 Processor] → [Wi-Fi Connectivity] → [IoT Cloud Platform] → [Web/Mobile Dashboard + Alerts]

This layered architecture ensures modularity, scalability, and real-time monitoring capabilities.

3.5 Hardware Components Used

The following hardware components were selected based on cost, compatibility, and performance:

Component	Model Used	Function

Microcontroller	ESP32	Central processor and Wi-Fi communication

Gas Sensor	MQ-Series	Detects toxic or flammable gases

Heart Rate Sensor	MAX30102	Measures pulse rate and oxygen variations

Temperature Sensor	MLX90614	Measures body temperature wirelessly

Power Supply	Rechargeable Battery	Portable power source

The compact size and lightweight nature of these components make them suitable for wearable integration.

3.6 Software Development

The software implementation involved two major parts:

Firmware Programming:

Written in Arduino IDE, including sensor calibration, data acquisition, wireless configuration, and alert parameters.

IoT Platform Configuration:

A cloud service such as Blynk, Firebase, goggle sheets or ThingSpeak was set up to receive, store, and display data in real time. The dashboard includes numerical values, trend charts, and alert indicators when thresholds are exceeded.

3.7 Data Flow Diagram (DFD)

To represent how data moves through the system, a Level-0 DFD is provided below.

[Worker → Sensors → ESP32 → IoT Cloud → Dashboard → Alerts → Supervisor]

This diagram shows that the communication loop continues as long as the device remains operational and connected to the internet.

3.7.1 System Block Diagram

A block diagram is used to illustrate the major hardware components and how they interact within the system. It provides a high-level view of the device architecture and signal processing pathway.

Gas Sensor

│

Heart Rate Sensor

│

Temp. Sensor

│

▼

ESP32 MCU

.                           ▼

|  |  | Processing + Wi-Fi Unit |  |  |
| --- | --- | --- | --- | --- |

▼           ▼

| Local Alerts |  | Cloud/IoT Platform |
| --- | --- | --- |

│

| (Buzzer/LED) |  |  |  |  | (Dashboard Storage) |  |
| --- | --- | --- | --- | --- | --- | --- |

▼                    ▼

| Worker Alerts |  | Supervisor/Mobile App |
| --- | --- | --- |

Explanation:

The block diagram shows the flow of data from the sensors to the ESP32 microcontroller, followed by wireless transmission to the IoT cloud platform. Local alerts (such as buzzer or LED indication) are triggered instantly when measured values exceed predefined thresholds. The cloud dashboard enables remote monitoring and visualization from a mobile or web interface.

3.7.2 System Workflow Flowchart

The flowchart below explains the operational sequence of the wearable device from startup to data transmission and alert handling.

Start Device

▼

Initialize ESP32 & Wi-Fi

▼

Sensor Initialization

| Read Sensor Values |  |
| --- | --- |

▼

Are The Values Above Safety Threshold?

| Yes | No |
| --- | --- |

▼

| │ Trigger Alert │ |  | │ Continue Normal Display│ |
| --- | --- | --- |

│ (Buzzer/Cloud)   │     └───────────────┬────────┘

└───────────┬─────┘                     ▼

|  | Send Data to IoT Server |
| --- | --- |

▼

| Update Dashboard |  |
| --- | --- |

▼

Loop and Continue

▼

Monitoring

Explanation:

The flowchart outlines how the system operates continuously in a loop. After booting, the ESP32 initializes the sensors and Wi-Fi module, collects readings, and checks whether values exceed safety limits. If unsafe conditions are detected, the system triggers alerts while sending data to the cloud platform. If values are normal, the system silently logs data without triggering warnings.

3.8 Prototype Assembly and Testing

The components were assembled on a flexible wearable strap to ensure comfort. Preliminary bench-testing was performed to verify individual sensor performance. Calibration adjustments were made based on test readings.

Field simulation testing was later conducted by:

Exposing the gas sensor to controlled emissions

Measuring heart rate during rest and mild physical activity

Testing temperature readings in normal and warm environments

All results were recorded and compared to expected values.

3.9 System Evaluation Criteria

The device was evaluated using the following performance measures:

Accuracy – how close sensor readings were to verified reference values

Response Time – time taken to detect and transmit data

Reliability – stability of reading output over repeated trials

Battery Efficiency – device runtime before requiring recharge

User Comfort – weight, size, and ease of wearing the device

These evaluation metrics helped assess the overall feasibility and improvement areas of the prototype.

3.10 Summary

This chapter explained the methods and processes used to develop the wearable IoT safety device. The approach included system design, component selection, firmware development, cloud integration, prototype assembly, and performance evaluation. The methodology ensures that the system functions as intended and can be replicated or upgraded in future research.

REFERENCES

Al-Fuqaha, A., Guizani, M., Mohammadi, M., Aledhari, M., & Ayyash, M. (2015). Internet of Things: A survey on enabling technologies, protocols, and applications. IEEE Communications Surveys & Tutorials.

Ashton, K. (2009). That ‘Internet of Things’ Thing. RFID Journal.

Creswell, J. W. (2018). Research Design: Qualitative, Quantitative, and Mixed Approaches. Sage.

Dimitrov, D. V. (2016). Medical Internet of Things and wearable health devices.

Journal of Medical Internet Research.

Fernandez, N., Romero, E., & Cruz, A. (2021). Wearable occupational safety devices: Challenges and solutions.

Journal of Safety Engineering.

Han, Z., Zhang, H., & Li, Q. (2019). Review of MQ gas sensor applications. Sensors and Actuators B.

Hussain, S., Ali, N., & Khan, M. (2022). IoT-enabled industrial monitoring systems: A systematic review.

IEEE Access.

Lopez, G., Sanz, P., & Garcia, M. (2018). Wearable sensing technologies for occupational risk prevention.

Safety Science Journal.

Madakam, S., Ramaswamy, R., & Tripathi, S. (2015). Internet of Things (IoT): A literature review.

Journal of Computer and Communications.

Occupational Safety and Health Administration (OSHA). (2022). Gas Safety Standards.

Pantelopoulos, A., & Bourbakis, N. (2010). A survey on wearable sensor-based systems for health monitoring. IEEE Transactions on Systems.

Park, S., & Jayaraman, S. (2020). The future of smart wearable health monitoring.

Biomedical Engineering Review.

Rajput, Y., Sharma, S., & Patel, R. (2022). Heart rate monitoring sensor comparison: A technical review.

International Journal of Engineering Science.

Raju, K., & Pitchumani, R. (2020). Human physiological responses during occupational stress.

Journal of Human Factors Engineering.

Sims, J., Adams, T., & Murphy, C. (2021). Non-contact wearable temperature technologies.

Biomedical Sensors Review.

World Health Organization. (2021). Occupational heat exposure and safety report.